| Concept from Paper | How You Used It in Your Work |
| --- | --- |
| Battery Level Monitoring | ➤ You introduced battery level tracking per step. ➤ Reroute logic to charging station if battery < threshold (20%). |
| Battery Threshold Management | ➤ Defined BATTERY_THRESHOLD and switched goal to charging station dynamically in A* if battery is low. |
| Task Reassignment Based on Battery Level | ➤ Modeled as battery-aware rerouting (low-battery robot seeks charging). In future, this can trigger task handover to another AGV. |
| Energy Modeling with Drive & Control Power | ➤ Incorporated P_DRIVE and P_CONTROL in your cost function for energy-optimized A*. ➤ Penalized turns (like the paper's use of control energy) to avoid unnecessary direction changes. |
| Workflow Optimization Using A* | ➤ Improved A* with energy as cost metric, not just distance. Similar to the paper's use of A* for shortest path with workflow efficiency. |
| Overhead Localization via Grid Mapping | ➤ Modeled environment as a 40x40 grid, simulating obstacle detection. Paper used overhead webcam for mapping; you used random obstacle generation. |
| Energy Efficiency Focus | ➤ Calculated Energy Efficiency Ratio (EER) and % energy savings similar to paper’s goal of reducing energy consumption and downtime. |
| Monitoring via GUI | ➤ While you didn’t use a GUI, you printed detailed battery and energy metrics, simulating a monitoring dashboard (paper used GUI for battery signals). |

🔹 Key Concepts Explained

1. Intermediate Charging

- AGVs don't wait for full discharge; they can top up their battery between tasks.

2. Displacement of Charging AGVs

- If a low-battery AGV needs to charge and a station is occupied by a higher battery AGV, the lower one can ask it to move.

- There’s a threshold difference: e.g., if the low-battery AGV is 25% lower than the occupying AGV, it gets priority.

🔹 Pearl Chain Method Formula

Used to decide if it’s worth it to displace an AGV:

C=DVS+DVD+MDC = D_{VS} + D_{VD} + MDC=DVS​+DVD​+MD

Where:

- DVSD_{VS}DVS​ = Distance the seeking AGV needs to travel to occupied charger

- DVDD_{VD}DVD​ = Distance the displaced AGV must travel to another charger

- MDMDMD = Extra distance/effort to maneuver out of the charger

Efficiency Conditions:

- Displacement allowed if battery difference ≥ 40%

- If difference 20–40%, only allow if total travel effort is lower than alternatives.

🔹 GAP (Generalized Assignment Problem)

Mathematically assigns AGVs to stations by:

- Minimizing total cost (distance, displacement penalties)

- Prioritizing low-battery AGVs by giving them lower costs for nearby chargers.

- Violations (like blocking low-battery AGVs) get huge penalties (10,000 units) to prevent bad allocations.